# 8/16 Channel H.264 Multiplex DVR

Features:

Advanced

■ 2 USB ports for mouse usage and backup.

■ Dual streaming enhances the speed of network transmission.

■ Built-in VGA output up to 1024x768 resolution.

- Individual setup of resolution, frame rate and video quality

for each channel.

■ Still image snapshot.

■ AVI converter with time stamp.

Basic

■ H.264 compression ideal for saving HDD space.

■ Real time live display.

■ Live display, record, backup, playback and network access simultaneously.

■ Picture-in-picture monitoring and 2X to 8X digital zoom display.

■ Control methods: front panel, USB mouse, IR remote controller, client viewer.

■ Intuitive GUI for easy configuration and menu driven operation.

■ Pan / Tilt / Zoom camera control.

■ Data backup: USB devices, network.

■ Event triggered with email notification: motion detection, alarm, video loss.

■ Supports Internet Explorer with same GUI as DVR site.

■ Multi-language OSD.

■ Central Management System (CMS) supports up to 15 DVRs.

■ 3G/GPRS mobile phone monitoring.

### Specifications:

| Model No. | D16-62CHV | D08-62CHV |
| --- | --- | --- |
| Video System | NTSC / PAL switch |  |
| Video Input | 16 CH BNC | 8 CH BNC |
| Video Input Level | 1.0 Vp-p Composite, 75 Ohm Balanced |  |
| Video Output | 1 CH BNC |  |
| SPOT Output | 1 CH BNC |  |
| VGA Output | 1024x768 |  |
| Audio Input | 4 CH RCA |  |
| Audio Input Level | 0.5~1.4 Vp-p @ 20K Ohm |  |
| Audio Output | 1CH RCA |  |
| Audio Backup | YES |  |
| Sensor | 16 input / 1 output | 8 input / 1 output |
| RS-485 | Pan / Tilt / Zoom camera |  |
| USB Port | 2 ports |  |
| Mouse | USB mouse |  |
| IR Remote Controller | YES |  |
| HDD Support | SATA HDD x 3 (Max. 1TB each) or SATA HDD x2 + DVD writer x1 | SATA HDD x2 (Max. 1TB each) + DVD writer x1 |
| DVD Writer | Optional |  |
| Display Division | 1,4,9,13,16 | 1,4,9 |
| GUI | YES |  |
| Title | 14 characters |  |
| PIP | YES |  |
| Digital Zoom | 2X ~ 8X |  |
| Video Compression | H.264 |  |
| Audio Compression | ADPCM |  |
| Resolution | 720x480, 720x240, 360x240 (NTSC) 720x576, 720x288, 360x288 (PAL) |  |
| Frame Rate | 120, 240, 480 (NTSC) 100, 200, 400 (PAL) | 60, 120, 240 (NTSC) 50, 100, 200 (PAL) |
| Quality | Highest, High, Normal, Basic, Below Basic |  |
| Record Mode | Manual, Schedule, Event (motion detection/sensor) |  |
| Pre-Alarm Recording | 16MB (10 sec) |  |
| Post-Alarm Recording | 1-999 sec |  |
| Motion Detection | 16X12 (sensitivity 1~15) |  |
| Search Mode | Time, Event |  |
| Playback Speed | Fast Forward/Backward 2X/4X/8X/16X/32X/64X Slow Forward/Backward 1/2X,1/4X,1/8X,1/16X Play/Pause |  |
| Backup Mode | USB flash driver / Network / Optional DVD media |  |
| Network Compression | H.264 |  |
| Software | YES |  |
| Internet Explorer | YES |  |
| CMS | YES |  |
| 3G Mobile Phone | Support |  |
| Network Users | 5 |  |
| Remote Notification | Email message |  |
| Service | TCP/IP, SMTP, DHCP, DDNS, PPPoE |  |
| Daylight Saving | #### YES |  |
| Password | 5 users including 1 administrator user authority selectable |  |
| Event List | 10000 |  |
| Log List | 10000 |  |
| Watchdog | Software Watchdog |  |
| Recovery | Auto-Reboot |  |
| Key Lock | YES |  |
| Firmware Update | USB device |  |
| Dimension | 430mm (W) x 70mm (H) x 360mm (D) |  |
| Power Source | DC12V |  |
| Operating Environment | 30 ~ 80% RH, 5°C ~ 40°C (41°F ~ 104°F) |  |