1/3 INCH COLOR CAMERA

# S P E C I F I C A T I O N S

1/3 INCH COLOR CAMERA

MODEL NO.: C3803-01B1 (NTSC) / C3804-01B1 (PAL) (DC12V)

C3803-01B2 (NTSC) / C3804-01B2 (PAL) (AC24V)

C3803-01B3 (NTSC) (AC110V) / C3804-01B4 (PAL) (AC230V)

C3803-01B5 (NTSC) / C3804-01B5 (PAL) (DUAL)

PICK UP DEVICE:                      1/3 INCH SONY CCD SENSOR

IMAGE SIZE:                           3.64 (H) x 4.82 (V) mm

PICTURE ELEMENTS:                  510 x 492 (NTSC)        500 x 582(PAL)

WHITE BALANCE:                      AUTO

BACK LIGHT COMPENSATION:         ON / OFF

GAIN CONTROL:                       AUTO

ELECTRONIC SHUTTER:                NTSC: 1/60s to 1/100,000s, PAL: 1/50s to 1/110,000s ON/OFF

SWITCHABLE

AUTO IRIS LENS CONTROL:            VIDEO OR DC DRIVE SWITCHABLE

POWER SOURCE:                      DC12V, AC 24V, 110V, 230V ±10%

OPTION: DUAL VOLT DC12/AC24V ±10%

POWER CONSUMPTION:               DC: 250mA (MAX.), AC: 3.2W (MAX.)

HORIZONTAL RESOLUTION:            420 TV LINES

SIGNAL NOISE RATIO:                 MORE THAN 48dB

VIDEO OUTPUT:                       1 Vp-p COMPOSITE VIDEO OUTPUT, 75 OHMS

MINIMUM ILLUMINATION:               0.02 LUX,  F=1.2

GAMMA CORRECTION:                 0.45

SYNC. SYSTEM:                       INTERNAL

LENS MOUNT:                         C & CS MOUNT

OPERATING TEMPERATURE:           -10°C  to  +50°C

DIMENSIONS (WxHxD):                 54x50x120mm

WEIGHT:                              560g